Enclosure no. 1 to SIWZ

As amended on 24.04.2017

Technical Specification

1.  Magnetron  for radiographic accelerator -2 pcs

| Magnetron (type, model, producer) | …………………………………………. |  |
| --- | --- | --- |
| Parameter | Required | Offered |
| Frequency range | 2993 to 3002MHz |  |
| Peak output power | Max 3,1MW |  |
| Microwave output | To no.10(WR284) waveguide |  |
| Type of cooling | water |  |
| Cathode type | Indirectly heated |  |
| Heater voltage | 14Vdc |  |
| Heater current | 7 – 9A |  |
| Anode voltage(peak) | 0 to 52kV |  |
| Anode current(peak) | 60 – 120A |  |
| Pulse duration | Max 5μs |  |
| Pulse repetition rate | 1 to 300Hz |  |
| Rate of rise of voltage pulse | 80 - 120kV/μs |  |
| Working temperature range | From 30 to 45C |  |
| Mounting position | any |  |
| Maximum weight | 9 kg in total |  |
| Magnetic field | 1000 – 1650gauss |  |

2.  Accessories:

| Name | Type, model, producer |
| --- | --- |
| Waveguide transition Dedicated for offered magnetron in point 1 -1 pc | ……………………………………. |
| Electromagnet of magnetron Mechanically proper with an offered magnetron in point 1 -1pc | …………………………………….. |

...........................................................
(signature of authorized person)